Vodafone USB Modem K3770

Code  Fetching a page

#include "mbed.h"

#include "VodafoneUSBModem.h"

#include "HTTPClient.h"

DigitalOut myled(LED1);

This needs to be changed to the sim card apn

Serial pc(USBTX, USBRX);

int main()

{

VodafoneUSBModem modem;

HTTPClient http;

char str[512];

int ret = modem.connect("mobile.o2.co.uk"); //pp.vodafone.co.uk");

if(ret)

{

pc.printf("Could not connect\n");

return;

}

//GET data

pc.printf("Trying to fetch page...\r\n");

ret = http.get("http://mbed.org/media/uploads/donatien/hello.txt", str, 128);

if (!ret)

{

pc.printf("Page fetched successfully - read %d characters \r\n", strlen(str));

pc.printf("Result: %s\r\n", str);

}

else

{

pc.printf("Error - ret = %d - HTTP return code = %d \r\n", ret, http.getHTTPResponseCode());

}

modem.disconnect();

}

Unlocking the Vodafone USB Modem K3770

This site has the details: http://modemunlock4u.blogspot.co.uk/2013/01/unlock-vodafone-huawei-modem-k3770.html

Firstly, find the IMEI number of your device and note it down. It will be required later.

NB: The IMEI number is located near to where the sim card should go for the K3770.

To unlock the modem, follow these instructions (to be done on a Windows PC):

- Plug the USB Modem into your PC and install the original Vodafone software that came with the USB Modem.

- Download the latest Huawei Dashboard from http://adf.ly/9jI7d or http://adf.ly/9jI9j. Alternatively, it can be found \Dropbox\e.quinox Data Logger\Datalogging 2012-2013\USB MODEM\UnlockVodafone.zip\ UTPS23.001.03.15.999.exe

- Install the Dashboard (UTPS23.001.03.15.999.exe)

- This will change the dashboard of the device. Note: changing the dashboard automatically terminates your warranty with the operator.

- Wait for the drivers to install and Run the new software (It will be called Mobile Broadband and will have a greed icon on the desktop if it is there)

- Download universal master code from here http://adf.ly/9jI27 or http://adf.ly/9jI43 and run. Alternatively, it can be found \Dropbox\e.quinox Data Logger\Datalogging 2012-2013\USB MODEM\UnlockVodafone.zip\Universal_MasterCode.exe

- Run the program Universal_MasterCode.exe.

- Switch to the Huawei Tab and type in your IMEI number and click Calculate.

- Copy the Unlock Code.

- Insert a new Sim card into the modem and open the dashboard i.e. Mobile Broadband which will have a greed icon on the desktop if it is there.

- It should ask you for the unlock code. Enter the unlock code and the device will be unlocked.